secure Network Expansion Proposal

By Ines Carpentier, Savannah Antoine

Table of Contents

Actual Situation	2

Intro	2

Inventory	2

Limitations	2

Solution/Recommendation	3

Cabling Infrastructure	3

Storage Improvements	3

Network Segmentation and VLANS	3

Addressing Scheme	4

VLSM Addressing	4

Wireless Connectivity	4

Security and VPN Connectivity	5

Cost Summary	5

Conclusions	5

Appendices	6

Appendix A: Terms and Definitions	6

Appendix B: Network Diagrams	7

Original Topology	7

Solution	7

Appendix C: Inventory List	8

Main Site	8

New Location Site	8

# Actual Situation

## Intro

Through conducting an interview with Health Life Specialists, it was determined that the hospital’s original site’s network infrastructure incorporated a mesh topology design as it consists of multiple direct connections between routers, switches, and the server to provide redundancy and maintain communication between departments and services. The next step was to design an optimized topology for the expansion of this medical facility based on the client’s wish in opening a second location approximately 12 km away from the main site.

## Inventory

The hospital’s main site’s inventory consisted of:

- Internet: 1Gbps Fiber

- Security: Cisco Firepower Firewall

- Switching: 2x 48-port Managed Switches

- Routing: 2x Routers (Internal and VoIP)

- IP Phones: 25 Devices

- Workstations: 35x Windows 11 Desktops

- Server: HP ProLiant (12 core, 32 GB RAM, 2TB RAID 10)

- Storage: 6TB NAS (RAID 5)

## Limitations

During the interview, several limitations in the network were identified. Staff at the new location will require constant access to the Patient Management System (PMS), imaging databases, and lab result portals hosted at the main site. Due to the hospital’s use of high-resolution medical imaging files, the current network experiences a lag during large file transmissions, and the storage capacity is becoming insufficient.

The client also indicated that all data and services must remain on-site due to strict medical privacy regulations and the large size of imaging files. Cloud-based solutions were rejected in favor of a locally managed infrastructure. In addition, the clients have requested for the hospital to have a secure wireless access for staff tablets, a separate guest Wi-Fi network for patients, and a VPN connection to support doctors working remotely on a hybrid schedule.

The expansion will also increase the number of employees by approximately 50%, creating a need for additional devices, improved bandwidth for data, secure inter-site communication, and scalable network segmentation using VLANS and VLSM.

# Solution/Recommendation

The proposed solution focuses on improving the hospital’s network performance, scalability, storage capacity, wireless connectivity, and security while ensuring compliance with medical privacy requirements. The upgraded network infrastructure will support the expansion to the new location while maintaining a stable and secure communication between both sites.

## Cabling Infrastructure

With the new location being approximately 12 km away from the main hospital site, fiber optic cabling will be implemented as the primary connection between both facilities. Fiber optic cables are more reliable over long distances than copper cabling and provide faster transmission speeds, lower latency, and improved security. This is essential for transmitting large medical imaging files and patient records between locations.

In addition, the existing cabling of the hospital’s topology will be upgraded to Shielded Twisted Pair (STP) CAT 6 cabling or higher, as CAT 6 STP supports transmission speeds up to 10Gbps, which will significantly reduce lag during large file transfers and improve communication between departments. Hospitals contain many electronic medical devices such as MRI machines, x-ray systems, and patient monitors which generate electromagnetic interferences (EMI). STP cabling will reduce the interferences and improve the overall network reliability.

## Storage Improvements

The hospital currently uses a 6 TB NAS configured with RAID 5 for data storage. Due to the increasing amount of high-resolution medical imaging files, the storage capacity is becoming insufficient.

To address this issue, an additional NAS device will be implemented to increase the available storage capacity and improve redundancy. This solution ensures that patient imaging files, medical records, and laboratory results remain securely stored on-site while reducing the risk of data loss in case of storage failure.

The RAID configuration will continue to provide fault tolerance and improve data availability. Expanding the NAS infrastructure will also allow future scalability as the hospital continues to grow.

## Network Segmentation and VLANS

The upgraded network will use VLAN’s to improve organization, security and manage traffic between departments. Each department will be assigned its own VLAN to reduce broadcast traffic and isolate sensitive medical data.

The following VLANS will be implemented:

### Addressing Scheme

| IP-Phones | VLAN 1 | 192.168.0.32 /27 | 255.255.255.224 |
| --- | --- | --- | --- |
| Doctors | VLAN 10 | 192.168.0.128 /28 | 255.255.255.240 |
| Nurses | VLAN 20 | 192.168.0.96 /27 | 255.255.255.224 |
| Admins/Reception | VLAN 30 | 192.168.0.144 /29 | 255.255.255.248 |
| Servers | VLAN 40 | 192.168.0.152 /29 | 255.255.255.248 |
| Lab-Techs | VLAN 50 | 192.168.0.64 /27 | 255.255.255.224 |
| IT | VLAN 60 | 192.168.0.160 /29 | 255.255.255.248 |
| Guests | VLAN 80 | 192.168.248.0 /21 | 255.255.248.0 |
| Tablets | VLAN 100 | 192.168.0.168 /29 | 255.255.255.248 |

Inter-VLAN routing will be configured to allow secure communication between departments when necessary. Separating departments into VLAN’s improves network efficiency and limits unauthorized access to sensitive systems.

### VLSM Addressing

Variable Length Subnet Masking (VLSM) will be implemented to efficiently allocate IP addresses based on the number of devices required in each department.

Using VLSM minimizes wasted IP addresses while allowing scalability for future expansion.

The guest wireless network will use a larger subnet to support a high number of patient devices connecting to the network at once.

### Wireless Connectivity

The hospital requires secure wireless connectivity for staff tablets and a separate guest network for patients. To meet this requirement, wireless access points will be installed throughout the facility, one for the staff wireless network and one for the guest wireless network.

##### Staff Wireless Network

The staff network will be password protected and connected to VLAN 100. This network will allow the staff to securely access medical systems and patient information from hospital-issued tablets.

##### Guest Wireless Network

A separate guest WI-FI network will be implemented using VLAN 80 and allowing up to 2046 IP addresses. This network will isolate patient devices from the hospital’s internal systems for security purposes. There’s no password as guests will be capable of freely connecting their devices to the network. DHCP services configured will automatically assign IP addresses to the connected devices, simplifying network management and reducing configuration errors.

### Security and VPN Connectivity

To strengthen security, the existing CISCO firewall will continue to be used along with a VPN tunnel. The secure tunnel will ensure that patient records, medical imaging files, and sensitive hospital data remain protected during transmission.

The VPN will also allow doctors to securely access hospital resources remotely from the recesses of their home. While the configured firewall will block unauthorized traffic and monitor suspicious activity attempting to enter the network.

## Cost Summary

The proposed expansion requires additional networking equipment and upgrades including:

- X1 Fiber optic cabling = $214.99

- X1 STP CAT 6 cabling= $495.68

- X1 Wireless access points=  $692.99

- X1 Additional NAS storage = $1018.94

- X1 Tablets = $859.99

- X1 Desktops = $1,639.99

- Networking hardware and installation materials

The estimated cost is approximately between $20,000 CAD and $60,000 CAD depending on the final hardware specifications, installation requirements, and number of devices purchased for staff.

# Conclusions

The proposed network solution successfully addresses the limitations identified during the client interview. By implementing an improved cabling infrastructure, expanded NAS storage, VLAN segmentation, secure wireless networks, and additional security with VPN and firewall, the hospital will be able to support its expansion while maintaining high performance and security standards.

The upgraded network topology will improve communication between hospital sites, reduce lag during large file transfers, and provide reliable access to medical systems and patient records. The implementation of VLANS and firewall security will also strengthen the network’s protection and improve traffic management throughout the hospital.

The estimated deployment time for the implementation is approximately several weeks depending on cabling installation and hardware configuration. The upgrades can be completed in phases to minimize disruptions and avoid shutting down hospital operations.

# Appendices

## Appendix A: Terms and Definitions

VLAN: A Virtual Local Area Network used to separate devices into different broadcast domains for improved security and traffic management

VPN: A Virtual Private Network that creates an encrypted connection between networks or remote users

NAS: Network Attached storage used for centralized file and data storage across a network

RAID: Redundant Array of Independent Disks used to improve data redundancy and storage performance

VLSM: Variable Length Subnet Masking used to efficiently divide IP address ranges into smaller subnets

Access Points: Networking devices that provide wireless connectivity to devices such as tablets and laptops

Network: A group of interconnected devices that communicate and share resources over the internet

Redundancy: The inclusion of backup connections or devices to maintain network operation during failures

Fiber Optic Cable: A high-speed cable that transmits data using light signals over long distances

Topology: The physical or logical layout of a network

EMI: Electromagnetic Interference caused by electronic devices disrupting network signals

CAT: Category rating for ethernet cabling that defines supported transmission speeds

STP: Shielded Twisted Pair cabling designed to reduce electromagnetic interference due to its extra layer of protection within it

Firewall: A security device that monitors and filters incoming and outgoing network traffic

Inter-Vlan Routing: A routing method allowing communication between separate VLAN’s

Server: A computer system that provides resources and services to devices on a network

DHCP: Dynamic Host Configuration Protocol used to automatically assign IP addresses to devices on a network

## Appendix B: Network Diagrams

### Original Topology

### Solution

## Appendix C: Inventory List

### Main Site

- Internet: 1Gbps Fiber

- Security: Cisco Firepower Firewall

- Switching: 2x 48-port Managed Switches

- Routing: 2x Routers (Internal and VoIP)

- IP Phones: 25 Devices

- Workstations: 35x Windows 11 Desktops

- Server: HP ProLiant (12 core, 32 GB RAM, 2TB RAID 10)

- Storage: 6TB NAS (RAID 5)

### New Location Site

Additions:

- Access Points

- Additional NAS

- Tablets

- Desktops

- Fiber Optic Cabling

- STP CAT 6

# Works Cited

“Cat6A 26AWG SSTP Solid Bulk Cable Stranded CMR/FT4 1000ft – Yellow.” PrimeCables.ca, PrimeCables, https://www.primecables.ca/p-359198-cab-bk-c6ast-4yls-cat6a-26awg-sstp-solid-bulk-cable-stranded-cmrft4-1000ft-yellow

“Cisco Meraki MR36 Wireless Access Point – Wi-Fi 6 – Cloud Managed.” CDW Canada, CDW, https://www.cdw.ca/product/cisco-meraki-mr36-wireless-access-point-wi-fi-6-cloud-managed/5974179

“Dell ECS1250 Slim Desktop Computer with Intel Core Ultra 5 225 Processor.” Best Buy Canada, Best Buy, https://www.bestbuy.ca/en-ca/product/dell-ecs1250-slim-desktop-computer-with-intel-core-ultra-5-225-processor-512gb-ssd-16-gb-ram-intel-uhd-graphics-windows-11-pro-platinum-silver/19362996

“Samsung 8" Galaxy Tab Active5 128GB Tablet.” Best Buy Canada, Best Buy, https://www.bestbuy.ca/en-ca/product/samsung-8-galaxy-tab-active5-128gb-tablet/18197142

“Synology DS925+ 4-Bay DiskStation NAS Server.” Staples Canada, Staples, https://www.staples.ca/products/3134852-en-synology-ds925-4-bay-diskstation-nas-server-amd-ryzen-v1500b-4-gb-ram

“Ubiquiti Fiber Optic Patch Network Cable – Fiber Optic for Network Device, Switch, Router.” Best Buy Canada, Best Buy, https://www.bestbuy.ca/en-ca/product/ubiquiti-fiber-optic-patch-network-cable-fiber-optic-for-network-device-switch-router-patch-